Q1. Concept of Network Topology and Types

Network Topology:
Network topology refers to the physical or logical arrangement of computers, cables, and network devices in a network.

Types of Network Topologies

1. Bus Topology

PC1 ─── PC2 ─── PC3 ─── PC4

|

Backbone Cable

Points

- All devices connect to a single backbone cable.

- Data travels in both directions in the cable.

- Terminators are placed at both ends.

- Simple and easy to install.

- Requires less cable than other topologies.

- Failure of backbone stops the entire network.

- Used in small networks.

2. Star Topology

PC1

|

PC2 ── Switch/Hub ── PC3

|

PC4

Points

- All devices connect to a central hub or switch.

- Data passes through the central device.

- Easy to install and manage.

- Failure of one node does not affect others.

- Central device failure stops the network.

- Easy to add or remove devices.

- Most commonly used topology today.

3. Ring Topology

PC1 ─── PC2

|       |

PC4 ─── PC3

Points

- Each device connects to two neighboring devices.

- Forms a closed loop (ring).

- Data moves in one direction around the ring.

- Uses token passing method.

- Equal access to all devices.

- Failure of one node can stop the network.

- Difficult to troubleshoot.

4. Mesh Topology

PC1────PC2

|\    /|

| \  / |

|  \/  |

|  /\  |

| /  \ |

PC3────PC4

Points

- Each device connects to every other device.

- Provides multiple paths for data transmission.

- Very reliable and fault tolerant.

- High data security.

- If one link fails, another path is used.

- Requires large number of cables.

- Very expensive to implement.

5. Tree Topology

Main Hub

|

─────────────

|            |

Hub1         Hub2

|  |         |  |

PC1 PC2     PC3 PC4

Points

- Combination of Star and Bus topology.

- Devices are arranged in hierarchical structure.

- Multiple star networks connected to backbone.

- Easy network expansion.

- Suitable for large organizations.

- Backbone failure affects entire network.

- Complex configuration.

Q2. Advantages and Disadvantages of Topologies

| Topology | Advantages | Disadvantages |
| --- | --- | --- |
| Bus | Low cost, easy installation, less cable | Backbone failure stops network, difficult troubleshooting |
| Star | Easy management, high performance, easy expansion | Central hub failure stops network, higher cost |
| Ring | Equal access, orderly data transfer | One node failure affects network, difficult to maintain |
| Mesh | Very reliable, high fault tolerance, secure | Very expensive, complex installation |

Comparison

| Feature | Bus | Star | Ring | Mesh |
| --- | --- | --- | --- | --- |
| Cost | Low | Medium | Medium | Very High |
| Scalability | Limited | High | Moderate | Low |
| Reliability | Low | High | Medium | Very High |
| Performance | Low when traffic increases | High | Moderate | Very High |

Q3. Hybrid Topology (Star + Bus)

Diagram

PC1   PC2

\   /

Switch1

|

========================   (Bus Backbone)

|

Switch2

/   \

PC3     PC4

Explanation

A Hybrid topology combines two or more network topologies.
Here Star networks are connected using a Bus backbone.

Advantages

- Combines benefits of multiple topologies.

- Improves network flexibility.

- Easier network expansion.

- Failure in one star network does not affect others.

- Better fault tolerance.

- Supports large organizations.

- Improves network efficiency and performance.